Micro Hardness Testing

I.	Introduction

+ Objective :

- Using the micro hardness tester machine

- Determining the hardness of material

+ Definition :

Micro Hardness Testing is a measure of how resistance solid matter is to 			various kinds of permanent shape change when a force is applied.

II.	Experiment

+ Apparatus :

- Micro hardness tester (SHIMADZU MICRO HARDNESS TESTERS)

- Specimen:	- Stainless steel

- Titanium

- Aluminum

- Carbon steel

Hardness tester 		Vickers hadness

+ Procedure :

- It is important that the polished specimens are also etched (lightly) to reveal the microstructural features.

- Measure the hardness at least in six different locations along longitudinal line or transversal line with indented space 1mm.

- There is scope for personal error in measuring the diagonal lengths in the Vickers hardness testing. Take a few measurements on the same indentation to get an accurate value of the length of the diagonal.

- It makes sure that the appropriate phase is chosen under the microscope and that the indentation is made in that area.

III.	Result

| Material | Diagonal Length I (μm) | Diagonal Length II (μm) | Average Diagonal Length (μm) | Hardness Number |
| --- | --- | --- | --- | --- |
| Stainless steel Titanium Aluminum alloy Carbon steel | 38.22 43.41 51.02 43.79 | 38.38 43.54 49.26 42.07 | 38.30 43.48 50.16 42.93 | 253 196 147 201 |

IV.	Conclusion

After completely experimental we can conclude that the specimens which have more hardness testing is stainless steel. Because , when the force applied we saw the diagonal length smaller than the other materials.

V.	Answer

1. What is hardness?

Answer: Hardness is a measure of how resistant solid matter is to various kinds of permanent 	shape change when a force applied.

2. What are the different ways in which hardness can be measured ?

Answer: There is scratch hardness , indentation hardness , and rebound hardness.

3. Name the different methods in which indentation hardness can be measured?

Answer: There are 	- Brinell hardness testing

- Rockwell hardness testing

- Vickers hardness testing

4. What is the different between macrohardness and microhardness ?

Answer:	 - Macrohardness is applied to tests with a larger test load.

- Microhardness is applied to tests with low test load.

5. What is the relationship between hardness and strength of material?

Answer: 	- Hardness study about measurement of material is to various kinds and shape 		changed.

- Strength of material study about behavior of solid subject to stress and strain.

6. How is the hardness of a material determined by hardness testing machine?

Answer: The most common way of measuring the hardness of material in a quantitative way is by indentation testing. In these case , an indenter is pressed at right angles into the surface of a material to be tested. The forcing of the indenter is done under controlling condition of load and rate of application.

7.

8. Compare hardness values

- By formula

HV = 0.1981 × Fd2

- Stainless steel 	:

HV = 0.1981 × 1.961(38.30×10-3)2

= 265

- Titanium		:

HV = 0.1981 × 1.961(43.48×10-3)2

= 205

- Aluminum alloys	:

HV = 0.1981 × 1.961(50.16×10-3)2

= 154

- Carbone steel	:

HV = 0.1981 × 1.916(42.93×10-3)2

= 206

- By machine

- Stainless steel 	:		HV = 253

- Titanium		:		HV = 196

- Aluminum alloys	:		HV = 147

- Carbone steel	:		HV = 201

Compare : After determine the result we saw that hardness value by using the formula is almost equal to hardness value by using machine.